PRAKASH CHEMICALS INTERNATIONAL

PRODUCT CATALOGUE — PART II

Products #35 to #112 | Scraped from pciplindia.com | Complete Technical Reference

| 78 Products in this Volume | #35–112 Continuation from Vol. 1 |
| --- | --- |

Plasticizers  •  PVC Stabilizers  •  Emulsifiers  •  Surfactants  •  Preservatives  •  UV Filters

Solvents  •  Antimicrobials  •  Epoxy Systems  •  Minerals  •  Food Ingredients  •  Micronutrients

Source: pciplindia.com  |  Compiled: April 2026  |  Contact: info@prakashchemicals.com  |  +91-265-6126000

# PRODUCTS INDEX (#35–112)

All 78 products in this volume are fully detailed with technical specifications. Source: pciplindia.com

| No. | Product Name | Category | CAS No. |
| --- | --- | --- | --- |
| 35 | Pine Oil | Natural Solvent / Disinfectant | 8002-09-3 |
| 36 | Dibutyl Phthalate (MAXFLEX 400 P N) | Plasticizer | 84-74-2 |
| 37 | Di Iso Butyl Phthalate (MAXFLEX 400 P I) | Plasticizer | 84-69-5 |
| 38 | Chlorinated Paraffin (MAXCHLOR) | Flame Retardant / Plasticizer | 85535-85-9 |
| 39 | Di Butyl Maleate (MAXFLEX 400 M) | Reactive Plasticizer / Monomer | 105-76-0 |
| 40 | Lead Oxide | PVC Stabilizer / Industrial Chemical | 1317-36-8 |
| 41 | Tribasic Lead Sulphate | PVC Stabilizer | 12202-17-4 |
| 42 | Dibasic Lead Phosphate | PVC Stabilizer / Lubricant | 15845-52-0 |
| 43 | One Pack Stabilizer | PVC Stabilizer System | Proprietary |
| 44 | Liquid Stabilizer | PVC Liquid Stabilizer | Mixture |
| 45 | Antistatic Agent | Polymer Additive | Mixture |
| 46 | Tin Stabilizer | PVC Stabilizer (High Performance) | Blend |
| 47 | Zinc Stearate | Lubricant / Stabilizer / Release Agent | 557-05-1 |
| 48 | Calcium Stearate | PVC Lubricant / Stabilizer / Food Additive | 1592-23-0 |
| 49 | Cocamido propyl Betaine (BETAINEMAX) | Amphoteric Surfactant | 61789-40-0 |
| 50 | Alkyl Polyglucoside (GLUCOMAX) | Non-ionic / Green Surfactant | 68515-73-1 |
| 51 | Zinc Sulphate | Micronutrient / Industrial Chemical | 7733-02-0 |
| 52 | Manganese Sulphate | Micronutrient / Industrial Chemical | 10034-96-5 |
| 53 | Precipitated Silica | Reinforcing Filler / Carrier / Adsorbent | 112926-00-8 |
| 54 | Polyglycerol Ester | Emulsifier / Food Additive | 67784-82-1 |
| 55 | Sorbitan Monostearate | Non-ionic Emulsifier (Span 60) | 1338-41-6 |
| 56 | Sorbitan Monooleate | Non-ionic Emulsifier (Span 80) | 1338-43-8 |
| 57 | Sorbitan Tristearate | Non-ionic Emulsifier (Span 65) | 26658-19-5 |
| 58 | VAM-VeoVa (Vinyl Acetate Monomer / Vinyl Ester of Versatic Acid) | Emulsion Polymer Monomer | 9/3/8002 |
| 59 | Poly Vinyl Acetate Homopolymer | Adhesive Polymer Emulsion | 9003-20-7 |
| 60 | Pure Acrylic Emulsion | Paint / Coating Binder | 25068-38-6 |
| 61 | Acrylic Styrene Emulsion | Paint / Coating Binder | 868765-93-9 |
| 62 | Phenalkamine Hardeners (MAXAMINE) | Epoxy Curing Agent / Hardener | 68410-23-1 |
| 63 | Polyamide Hardeners (MAXAMIDE) | Epoxy Curing Agent / Hardener | Blend |
| 64 | Epoxy Resin (EPOMAX) | Thermoset Resin | 25068-38-6 |
| 65 | Phenoxyethanol | Preservative / Cosmetic Ingredient | 122-99-6 |
| 66 | Chlorhexidine Gluconate (MAXICIDE) | Broad-spectrum Antimicrobial / Disinfectant | 18472-51-0 |
| 67 | Monochloro Acetic Acid (MCAA) | Chemical Intermediate | 79-11-8 |
| 68 | Sodium Monochloro Acetate (SMCA) | Chemical Intermediate | 3926-62-3 |
| 69 | Dimethyl Amino Propyl Amine (DMAPA) | Amine / Chemical Intermediate | 109-55-7 |
| 70 | Benzyl Alcohol | Solvent / Preservative / Fragrance Ingredient | 100-51-6 |
| 71 | Benzyl Chloride | Chemical Intermediate / Chlorinating Agent | 100-44-7 |
| 72 | Castor Oil Ethoxylates | Non-ionic Emulsifier / Solubilizer | 61791-12-6 |
| 73 | Nonyl Phenol Ethoxylates (NPE) | Non-ionic Surfactant (Industrial) | 9016-45-9 |
| 74 | Polysorbate 80 (Tween 80) | Non-ionic Emulsifier (O/W) | 9005-65-6 |
| 75 | Polysorbate 60 (Tween 60) | Non-ionic Emulsifier (O/W) | 9005-67-8 |
| 76 | Polysorbate 20 (Tween 20) | Non-ionic Emulsifier / Solubilizer | 9005-64-5 |
| 77 | Acid Thickener (Carbopol-type / HASE thickener) | Rheology Modifier / Thickener | 9003-01-4 / Blend |
| 78 | Piroctone Olamine | Anti-Dandruff Active / Biocide | 68890-66-4 |
| 79 | Triclosan | Antimicrobial / Preservative | 3380-34-5 |
| 80 | Propyl Paraben | Cosmetic / Pharmaceutical Preservative | 94-13-3 |
| 81 | PROMIX (Methylisothiazolinone / Methylchloroisothiazolinone blend) | Biocide / Preservative | Mixture |
| 82 | Polyethylene Glycol (PEG) | Humectant / Solubilizer / Industrial Lubricant | 25322-68-3 |
| 83 | Para Chloro Meta Xylenol (PCMX) | Antiseptic / Antimicrobial | 88-04-0 |
| 84 | Octyl Methoxy Cinnamate (OMC) | UV Filter / Sunscreen Active | 5466-77-3 |
| 85 | Methyl Paraben | Cosmetic / Pharmaceutical Preservative | 99-76-3 |
| 86 | Lauryl Amine Oxide (LAO) | Amphoteric Surfactant / Foam Booster | 1643-20-5 |
| 87 | Glyceryl Monostearate (GMS) | Food Emulsifier / Cosmetic Emulsifier | 123-94-4 |
| 88 | Enzymes (Industrial) | Biological Catalyst / Detergent Additive | Various |
| 89 | DMDM Hydantoin | Formaldehyde-releasing Cosmetic Preservative | 6440-58-0 |
| 90 | Cetyl Trimethyl Ammonium Chloride (CTAC) | Cationic Surfactant / Hair Conditioner | 112-02-7 |
| 91 | Cocodiethanolamide (CDEA) | Non-ionic Foam Booster / Viscosifier | 61791-31-9 |
| 92 | Carbomer (Carbopol) | Rheology Modifier / Gelling Agent | 9003-01-4 |
| 93 | Benzophenone-4 (BP-4) | UV Filter / Sunscreen Active | 4065-45-6 |
| 94 | Benzophenone-3 (Oxybenzone) | UV Filter / Sunscreen Active (UVA+UVB) | 131-57-7 |
| 95 | Benzalkonium Chloride (BKC) | Cationic Biocide / Disinfectant / Preservative | 8001-54-5 |
| 96 | Butylated Hydroxytoluene (BHT) | Antioxidant / Preservative | 128-37-0 |
| 97 | Calcium Hypochlorite | Disinfectant / Sanitizer / Bleaching Agent | 7778-54-3 |
| 98 | Formaldehyde | Chemical Intermediate / Biocide / Fixative | 50-00-0 |
| 99 | Bleaching Earth (Activated Clay / Fuller's Earth) | Adsorbent / Bleaching Agent | 14808-60-7 |
| 100 | Phosphoric Acid | Inorganic Acid / Food Additive | 7664-38-2 |
| 101 | Maize Starch | Food Ingredient / Industrial Binder | 9005-25-8 |
| 102 | Liquid Glucose (Corn Syrup) | Food Sweetener / Industrial Binder | 8029-43-4 |
| 103 | Talc | Filler / Lubricant / Cosmetic Ingredient | 14807-96-6 |
| 104 | Kaolin (China Clay) | Filler / Pigment Extender / Industrial Mineral | 1332-58-7 |
| 105 | Petroleum Jelly (Petrolatum) | Emollient / Cosmetic / Industrial Lubricant | 8009-03-8 |
| 106 | White Mineral Oil (Liquid Paraffin) | Cosmetic / Food / Pharmaceutical / Industrial Lubricant | 8042-47-5 |
| 107 | Toluene Diisocyanate (TDI) | Polyurethane Raw Material / Isocyanate | 584-84-9 |
| 108 | Soap Noodles | Soap Raw Material | Mixture |
| 109 | Methylene Chloride (DCM) | Chlorinated Solvent | 75-09-2 |
| 110 | Dextrose Monohydrate | Food / Pharmaceutical / Fermentation Ingredient | 5996-10-1 |
| 111 | Citric Acid | Organic Acid / Food Additive / Chelating Agent | 5949-29-1,77-92-9 |
| 112 | Cetostearyl Alcohol (CSA) | Cosmetic Co-emulsifier / Thickener / Emollient | 67762-27-0 |

# PRODUCT DETAILS (#35–112)

Full technical specifications for each of the 78 products, including formula, physical properties, applications, and storage.

| 35. Pine Oil [Natural Solvent / Disinfectant] |  |
| --- | --- |
| Chemical Formula | C₁₀H₁₈O (mainly α-terpineol) |
| HS Code | 38059090 |
| CAS Number | 8002-09-3 |
| Appearance | Clear to pale yellow liquid |
| Odour | Strong pine/terpene characteristic |
| Density | 0.924–0.935 g/cm³ |
| Boiling Point | 217°C |
| Melting Point | N/A |
| Flash Point | 65°C |
| Solubility | Partially soluble in water; miscible with alcohols |
| pH | N/A |
| Purity / Grade | ≥65% alpha-terpineol (industrial grade) |
| Packaging | 210 L drums |
| Description | Pine Oil is a natural terpene-based solvent and disinfectant derived from pine wood pulp. It is widely used as a fragrance component, cleaning agent, flotation reagent in mining, and antimicrobial in household cleaners. |
| Applications | Disinfectant & cleaning agents; flotation agent in mineral processing; fragrance in floor cleaners; textile scouring; printing ink solvent; adhesives |
| Storage | Store in cool, well-ventilated area away from heat. Keep containers sealed. |

| 36. Dibutyl Phthalate (MAXFLEX 400 P N) [Plasticizer] |  |
| --- | --- |
| Chemical Formula | C₁₆H₂₂O₄ |
| HS Code | 29173910 |
| CAS Number | 84-74-2 |
| Appearance | Clear, oily, colourless liquid |
| Odour | Faint, mild odour |
| Density | 1.043–1.048 g/cm³ |
| Boiling Point | 340°C |
| Melting Point | -35°C |
| Flash Point | 157°C |
| Solubility | Insoluble in water; miscible with organic solvents |
| pH | N/A |
| Purity / Grade | ≥99.5% |
| Packaging | 215 kg drums / IBC |
| Description | Dibutyl Phthalate (DBP) is a primary plasticizer for PVC and cellulosic polymers, providing flexibility and workability. It is also used as a solvent for dyes, adhesives, and printing inks. |
| Applications | PVC plasticizer; cellulose acetate plasticizer; adhesives & sealants; nitrocellulose lacquers; pesticide carrier; cosmetic nail polish (legacy use) |
| Storage | Store in sealed containers in cool, well-ventilated area. Avoid prolonged skin contact. |

| 37. Di Iso Butyl Phthalate (MAXFLEX 400 P I) [Plasticizer] |  |
| --- | --- |
| Chemical Formula | C₁₆H₂₂O₄ |
| HS Code | 29173910 |
| CAS Number | 84-69-5 |
| Appearance | Clear, oily liquid |
| Odour | Faint characteristic odour |
| Density | 1.038–1.042 g/cm³ |
| Boiling Point | 327°C |
| Melting Point | -55°C |
| Flash Point | 166°C |
| Solubility | Insoluble in water; miscible with organics |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 215 kg drums |
| Description | Di-Isobutyl Phthalate (DIBP) is a plasticizer used as an alternative to DBP, offering similar compatibility with PVC and cellulose derivatives, with slightly better low-temperature performance. |
| Applications | PVC & cellulose resin plasticizer; adhesives; nitrocellulose coatings; paper & leather coatings; printing inks |
| Storage | Store in sealed containers in cool, ventilated area. Avoid ignition sources. |

| 38. Chlorinated Paraffin (MAXCHLOR) [Flame Retardant / Plasticizer] |  |
| --- | --- |
| Chemical Formula | CₙH(2n+2-x)Clₓ |
| HS Code | 29039940 |
| CAS Number | 85535-85-9 |
| Appearance | Light yellow to amber viscous liquid |
| Odour | Slight chlorine odour |
| Density | 1.15–1.35 g/cm³ (varies by chlorine content) |
| Boiling Point | >300°C |
| Melting Point | N/A |
| Flash Point | >200°C |
| Solubility | Insoluble in water; soluble in hydrocarbons and chlorinated solvents |
| pH | N/A |
| Purity / Grade | Chlorine content: 40–70% (as per grade) |
| Packaging | 200 L drums / IBC |
| Description | Chlorinated Paraffins are used as secondary plasticizers, extreme-pressure lubricant additives, and flame retardants in PVC, rubber, and metalworking fluids. |
| Applications | PVC secondary plasticizer; flame retardant in cables & plastics; metalworking cutting fluids; rubber compounds; sealants & adhesives |
| Storage | Store in sealed containers. Avoid exposure to strong acids and high temperatures. |

| 39. Di Butyl Maleate (MAXFLEX 400 M) [Reactive Plasticizer / Monomer] |  |
| --- | --- |
| Chemical Formula | C₁₂H₂₀O₄ |
| HS Code | 29171990 |
| CAS Number | 105-76-0 |
| Appearance | Clear, colourless liquid |
| Odour | Mild ester odour |
| Density | 1.050–1.055 g/cm³ |
| Boiling Point | 281°C |
| Melting Point | -85°C |
| Flash Point | 142°C |
| Solubility | Insoluble in water; miscible with organic solvents |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 215 kg drums |
| Description | Di Butyl Maleate (DBM) is a reactive plasticizer and co-monomer used in polymer synthesis, especially for acrylic and PVC-based systems, improving flexibility and compatibility. |
| Applications | PVC internal plasticizer; co-monomer in acrylic polymers; adhesives; UV-curable coatings; release coatings |
| Storage | Store away from heat and oxidizing agents. Mild skin irritant — use PPE. |

| 40. Lead Oxide [PVC Stabilizer / Industrial Chemical] |  |
| --- | --- |
| Chemical Formula | PbO / Pb₃O₄ |
| HS Code | 28241010 |
| CAS Number | 1317-36-8 |
| Appearance | Yellow to red-orange powder |
| Odour | Odourless |
| Density | 9.53 g/cm³ (litharge, PbO) |
| Boiling Point | 1477°C |
| Melting Point | 888°C |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water; soluble in acids and alkalis |
| pH | N/A |
| Purity / Grade | ≥98% PbO |
| Packaging | 25 kg bags |
| Description | Lead Oxide (litharge/minium) is used as a heat and light stabilizer in PVC compounds, as a pigment, and in the manufacture of glass, ceramics, and lead batteries. |
| Applications | PVC heat stabilizer (legacy); glass & ceramic colorant; lead-acid battery plates; rust-inhibiting primers; soldering fluxes |
| Storage | Toxic heavy metal compound — handle with full PPE. Store in sealed containers away from food sources. Follow local regulations. |

| 41. Tribasic Lead Sulphate [PVC Stabilizer] |  |
| --- | --- |
| Chemical Formula | 3PbO·PbSO₄·H₂O |
| HS Code | 28332990 |
| CAS Number | 12202-17-4 |
| Appearance | White powder |
| Odour | Odourless |
| Density | 6.92 g/cm³ |
| Boiling Point | N/A (decomposes) |
| Melting Point | >500°C (decomposes) |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water |
| pH | N/A |
| Purity / Grade | ≥99% |
| Packaging | 25 kg bags |
| Description | Tribasic Lead Sulphate is a traditional heat stabilizer and light stabilizer for rigid PVC applications, providing excellent long-term thermal stability in pipes, profiles, and wire insulation. |
| Applications | Rigid PVC heat stabilizer; PVC pipes, profiles & cables; flexible PVC compounds |
| Storage | Lead compound — hazardous. Use full PPE. Store sealed away from food. Comply with local lead handling regulations. |

| 42. Dibasic Lead Phosphate [PVC Stabilizer / Lubricant] |  |
| --- | --- |
| Chemical Formula | 2PbO·PbHPO₄ |
| HS Code | 28351090 |
| CAS Number | 15845-52-0 |
| Appearance | White powder |
| Odour | Odourless |
| Density | ~6.9 g/cm³ |
| Boiling Point | N/A |
| Melting Point | ~700°C |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water |
| pH | N/A |
| Purity / Grade | ≥98% |
| Packaging | 25 kg bags |
| Description | Dibasic Lead Phosphate (DBLP) is a PVC heat and light stabilizer with combined lubricating properties, used in rigid and semi-rigid PVC formulations. |
| Applications | PVC heat & light stabilizer; rigid PVC pipes, profiles, cables; internal lubricant in PVC compounds |
| Storage | Lead compound — toxic. Full PPE required. Store sealed, away from food. Comply with regulations. |

| 43. One Pack Stabilizer [PVC Stabilizer System] |  |
| --- | --- |
| Chemical Formula | Proprietary blend |
| HS Code | 38123990 |
| CAS Number | Proprietary |
| Appearance | White to off-white powder or granules |
| Odour | Faint characteristic |
| Density | ~1.3–1.8 g/cm³ |
| Boiling Point | N/A |
| Melting Point | N/A |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water |
| pH | N/A |
| Purity / Grade | Per specification |
| Packaging | 25 kg bags / boxes |
| Description | One-Pack Stabilizer systems are pre-blended PVC thermal stabilizer packages combining heat stabilizers, lubricants, and process aids in a single additive for ease of use in rigid PVC applications. |
| Applications | Rigid PVC pipes, profiles, fittings; window frames; cable compounds; foam PVC sheets |
| Storage | Store in dry, sealed containers at ambient temperature. Avoid humidity. |

| 44. Liquid Stabilizer [PVC Liquid Stabilizer] |  |
| --- | --- |
| Chemical Formula | Proprietary blend (Ba/Zn or Ca/Zn based) |
| HS Code | 38123990 |
| CAS Number | Mixture |
| Appearance | Clear to amber liquid |
| Odour | Mild characteristic odour |
| Density | 0.9–1.2 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | N/A |
| Flash Point | >80°C |
| Solubility | Insoluble in water; compatible with PVC/plasticizer systems |
| pH | N/A |
| Purity / Grade | Per specification |
| Packaging | 200 L drums / IBC |
| Description | Liquid PVC Stabilizer (typically Ba/Zn or Ca/Zn systems) provides heat and light stability in flexible PVC applications and is easy to incorporate in paste/calendering processes. |
| Applications | Flexible PVC flooring, wall coverings; PVC plastisols; artificial leather; cables; film & sheet |
| Storage | Store sealed in cool, dry area. Avoid contamination with water. Follow SDS for specific product. |

| 45. Antistatic Agent [Polymer Additive] |  |
| --- | --- |
| Chemical Formula | Proprietary (quaternary ammonium salt / glycerol monostearate based) |
| HS Code | 29239000 |
| CAS Number | Mixture |
| Appearance | White to off-white solid or liquid |
| Odour | Faint characteristic odour |
| Density | ~0.9–1.1 g/cm³ |
| Boiling Point | N/A |
| Melting Point | Varies |
| Flash Point | >100°C |
| Solubility | Partially soluble/dispersible in water; compatible with polymers |
| pH | 6–8 (aqueous dispersion) |
| Purity / Grade | Per specification |
| Packaging | 25 kg bags (solid) / 200 kg drums (liquid) |
| Description | Antistatic Agents are additives incorporated into plastics (PE, PP, PVC) to reduce static charge build-up, preventing dust attraction, packaging cling, and electrostatic hazards. |
| Applications | Polyolefin & PVC antistatic additive; packaging films; agricultural films; electronic component packaging; flooring |
| Storage | Store in cool, dry, sealed conditions. Solid grades: protect from moisture. |

| 46. Tin Stabilizer [PVC Stabilizer (High Performance)] |  |
| --- | --- |
| Chemical Formula | Organotin compound (dibutyltin/dioctyltin based) |
| HS Code | 29332990 |
| CAS Number | Blend |
| Appearance | Clear liquid or white powder |
| Odour | Characteristic organotin odour |
| Density | 1.05–1.18 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | N/A |
| Flash Point | >100°C |
| Solubility | Insoluble in water; compatible with PVC |
| pH | N/A |
| Purity / Grade | Tin content: 17–19% (typical) |
| Packaging | 25 kg cans / 200 L drums |
| Description | Organotin (dibutyltin/dioctyltin) stabilizers are high-performance PVC thermal stabilizers offering exceptional clarity, weatherability, and heat resistance — the preferred stabilizer for transparent and food-contact PVC. |
| Applications | Transparent rigid PVC (bottles, blister packs); food-contact PVC (dioctyltin grade); PVC window profiles; rigid pipes; film and sheet |
| Storage | Store in sealed containers. Organotin compounds are toxic — use full PPE. Follow local regulations. |

| 47. Zinc Stearate [Lubricant / Stabilizer / Release Agent] |  |
| --- | --- |
| Chemical Formula | Zn(C₁₇H₃₅COO)₂ |
| HS Code | 29157090 |
| CAS Number | 557-05-1 |
| Appearance | White, fine, free-flowing powder |
| Odour | Faint fatty odour |
| Density | 1.09 g/cm³ |
| Boiling Point | Decomposes >300°C |
| Melting Point | 120–130°C |
| Flash Point | N/A (solid) |
| Solubility | Insoluble in water; soluble in hot aromatic solvents |
| pH | N/A |
| Purity / Grade | ≥99% as zinc stearate |
| Packaging | 25 kg bags |
| Description | Zinc Stearate is a metallic soap used as an internal/external lubricant, mold release agent, and co-stabilizer in PVC and rubber processing. Also used as a flatting agent in paints. |
| Applications | PVC lubricant & co-stabilizer; rubber processing lubricant; mold release agent; paint matting/flatting agent; cosmetics (face powder); paper coating |
| Storage | Store in cool, dry area in sealed bags. Avoid moisture absorption. |

| 48. Calcium Stearate [PVC Lubricant / Stabilizer / Food Additive] |  |
| --- | --- |
| Chemical Formula | Ca(C₁₇H₃₅COO)₂ |
| HS Code | 29157090 |
| CAS Number | 1592-23-0 |
| Appearance | White, fine, free-flowing powder |
| Odour | Faint fatty odour |
| Density | 1.03–1.08 g/cm³ |
| Boiling Point | >400°C |
| Melting Point | 155–160°C |
| Flash Point | >250°C (solid) |
| Solubility | Insoluble in water |
| pH | N/A |
| Purity / Grade | ≥99% |
| Packaging | 25 kg bags |
| Description | Calcium Stearate is a versatile metallic soap used as PVC lubricant and co-stabilizer, anti-caking agent in food/pharma, mold release agent in rubber, and surface coating for paper. |
| Applications | PVC lubricant & co-stabilizer; food anti-caking agent (E470); pharmaceutical tablet lubricant; rubber processing aid; paper & board coating; cosmetics |
| Storage | Store in sealed bags in dry, cool conditions. Non-hazardous under normal handling. |

| 49. Cocamido propyl Betaine (BETAINEMAX) [Amphoteric Surfactant] |  |
| --- | --- |
| Chemical Formula | C₁₉H₃₈N₂O₃ |
| HS Code | 34024200 |
| CAS Number | 61789-40-0 |
| Appearance | Clear to slightly hazy amber liquid (30–35% solution) |
| Odour | Faint fatty/soapy odour |
| Density | 1.040–1.060 g/cm³ |
| Boiling Point | >100°C |
| Melting Point | <0°C |
| Flash Point | Non-flammable |
| Solubility | Miscible with water |
| pH | 4.5–6.0 (neat); 5.0–7.0 (10% solution) |
| Purity / Grade | ≥30% active matter |
| Packaging | 200 kg drums / IBC |
| Description | Cocamidopropyl Betaine (CAPB) is a mild amphoteric surfactant derived from coconut oil. It is widely used as a foam booster, viscosity builder, and mildness enhancer in personal care and home care products, compatible with both anionic and cationic surfactants. |
| Applications | Shampoos & hair conditioners; liquid soaps & body wash; baby care products; facial cleansers; bubble baths; mild dishwashing liquids; industrial cleaning |
| Storage | Store at 10–35°C in sealed containers. May become hazy at low temperatures — reversible on warming. |

| 50. Alkyl Polyglucoside (GLUCOMAX) [Non-ionic / Green Surfactant] |  |
| --- | --- |
| Chemical Formula | CₙH(2n+1)O(C₆H₁₀O₅)ₘOH |
| HS Code | 3402.13.00 |
| CAS Number | 68515-73-1 |
| Appearance | Clear to slightly yellow viscous liquid |
| Odour | Faint fatty/sweet odour |
| Density | 1.08–1.12 g/cm³ |
| Boiling Point | >100°C |
| Melting Point | N/A |
| Flash Point | Non-flammable |
| Solubility | Fully miscible with water |
| pH | 11.0–12.0 (neat); 6.0–9.0 (diluted) |
| Purity / Grade | ≥50–60% active matter |
| Packaging | 200 kg drums / IBC |
| Description | Alkyl Polyglucosides (APG) are naturally derived, biodegradable non-ionic surfactants produced from glucose and fatty alcohols. They are mild, skin-compatible, and highly effective across a wide pH range. |
| Applications | Mild shampoos & skin cleansers; baby products; household cleaners; dish detergents; industrial & institutional cleaners; agricultural formulations (adjuvants) |
| Storage | Store in sealed containers at 15–35°C. Avoid freezing. Keep away from strong acids. |

| 51. Zinc Sulphate [Micronutrient / Industrial Chemical] |  |
| --- | --- |
| Chemical Formula | ZnSO₄·H₂O / ZnSO₄·7H₂O |
| HS Code | 28332940 |
| CAS Number | 7733-02-0 |
| Appearance | White crystalline powder (monohydrate) or colorless crystals (heptahydrate) |
| Odour | Odourless |
| Density | 3.54 g/cm³ (monohydrate) |
| Boiling Point | Decomposes >500°C |
| Melting Point | 100°C (monohydrate, loses water) |
| Flash Point | Non-flammable |
| Solubility | 57.7 g/100 mL (20°C) in water |
| pH | 4.0–6.0 (5% solution) |
| Purity / Grade | ≥35% Zn (monohydrate) |
| Packaging | 25 kg bags / 50 kg bags |
| Description | Zinc Sulphate is a key zinc micronutrient for agriculture, used in foliar sprays and soil application for zinc-deficient crops. Also used in animal feed, water treatment, and electroplating. |
| Applications | Agricultural micronutrient (zinc fertilizer); animal feed supplement; water treatment (algicide); electroplating; rayon fiber production; zinc-based chemicals |
| Storage | Store in sealed bags away from moisture. Non-hazardous under normal conditions. |

| 52. Manganese Sulphate [Micronutrient / Industrial Chemical] |  |
| --- | --- |
| Chemical Formula | MnSO₄·H₂O |
| HS Code | 28332990 |
| CAS Number | 10034-96-5 |
| Appearance | Pale pink to white crystalline powder |
| Odour | Odourless |
| Density | 3.25 g/cm³ |
| Boiling Point | 450°C (decomposes) |
| Melting Point | 700°C (anhydrous) |
| Flash Point | Non-flammable |
| Solubility | 63 g/100 mL (20°C) in water |
| pH | 6.0–7.5 (5% solution) |
| Purity / Grade | ≥31.8% Mn (monohydrate) |
| Packaging | 25 kg bags |
| Description | Manganese Sulphate Monohydrate is an essential manganese micronutrient for crops and animal feed. It is also used as a catalyst, in electroplating, and in the manufacture of dry cell batteries. |
| Applications | Crop micronutrient fertilizer; animal feed additive; dry cell batteries (MnO₂ production); electroplating; paint drier; fungicide |
| Storage | Store in sealed bags in dry conditions. Non-hazardous under standard handling. |

| 53. Precipitated Silica [Reinforcing Filler / Carrier / Adsorbent] |  |
| --- | --- |
| Chemical Formula | SiO₂·nH₂O |
| HS Code | 28112200 |
| CAS Number | 112926-00-8 |
| Appearance | White, fine amorphous powder |
| Odour | Odourless |
| Density | ~2.0 g/cm³ (true); ~0.1–0.2 g/cm³ (bulk) |
| Boiling Point | N/A |
| Melting Point | >1600°C |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water; soluble in HF and strong alkali |
| pH | 6.0–8.0 (4% aqueous suspension) |
| Purity / Grade | ≥98% SiO₂ (dry basis) |
| Packaging | 25 kg paper bags |
| Description | Precipitated Silica is an amorphous, white, porous silica powder produced by chemical precipitation. It is the key reinforcing agent in green tyre rubber, and also used as an adsorbent, anti-caking agent, and thickener. |
| Applications | Green tyre & rubber reinforcing filler; toothpaste (abrasive & thickener); anti-caking agent in food & feed; paint flatting/matting agent; silica gel/desiccant carrier; beer clarifier |
| Storage | Store in sealed bags in dry, cool conditions. Avoid inhalation of fine dust — use respiratory protection. |

| 54. Polyglycerol Ester [Emulsifier / Food Additive] |  |
| --- | --- |
| Chemical Formula | Polyglycerol fatty acid ester |
| HS Code | 34029090 |
| CAS Number | 67784-82-1 |
| Appearance | Light yellow to amber viscous liquid or soft solid |
| Odour | Faint fatty odour |
| Density | 1.0–1.15 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | Varies (25–65°C depending on grade) |
| Flash Point | >200°C |
| Solubility | Dispersible/emulsifiable in water; soluble in fats and oils |
| pH | 4.0–7.0 (10% dispersion) |
| Purity / Grade | ≥99% ester content |
| Packaging | 25 kg bags (solid) / 200 kg drums (liquid) |
| Description | Polyglycerol Esters (PGE) are non-ionic emulsifiers used in food, cosmetics, and industrial applications. They improve aeration, texture, shelf life, and emulsification in bakery, dairy, and confectionery products. |
| Applications | Food emulsifier (E475/E476) in bakery, margarine, chocolate; cosmetic emulsifier; non-dairy creamer; crystal modifier in fats; personal care formulations |
| Storage | Store in sealed containers at cool, dry conditions. Solid grades should be protected from moisture. |

| 55. Sorbitan Monostearate [Non-ionic Emulsifier (Span 60)] |  |
| --- | --- |
| Chemical Formula | C₂₄H₄₆O₆ |
| HS Code | 34029090 |
| CAS Number | 1338-41-6 |
| Appearance | Cream to tan waxy solid |
| Odour | Faint fatty odour |
| Density | 0.975–1.0 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | 54–57°C |
| Flash Point | >200°C |
| Solubility | Dispersible in warm water; soluble in most organic solvents |
| pH | 5.0–7.5 (3% dispersion) |
| Purity / Grade | ≥97% (as sorbitan monostearate) |
| Packaging | 25 kg bags / drums |
| Description | Sorbitan Monostearate (Span 60) is a lipophilic non-ionic emulsifier used in food, pharmaceuticals, and cosmetics as a W/O emulsifier, crystal modifier, and stabilizer. |
| Applications | Food W/O emulsifier (E491); pharmaceutical emulsifier; cosmetic cream formulations; confectionery (chocolate bloom inhibitor); yeast rehydration agent |
| Storage | Store below 30°C away from moisture. Protect from direct sunlight. |

| 56. Sorbitan Monooleate [Non-ionic Emulsifier (Span 80)] |  |
| --- | --- |
| Chemical Formula | C₂₄H₄₄O₆ |
| HS Code | 34029090 |
| CAS Number | 1338-43-8 |
| Appearance | Amber, oily liquid |
| Odour | Faint fatty odour |
| Density | 0.99–1.01 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | <10°C |
| Flash Point | >200°C |
| Solubility | Dispersible in warm water; soluble in oils and organics |
| pH | 5.0–7.5 (3% dispersion) |
| Purity / Grade | ≥97% |
| Packaging | 200 kg drums |
| Description | Sorbitan Monooleate (Span 80) is a highly lipophilic emulsifier used in W/O emulsions, agrochemical formulations, and industrial applications as a wetting and dispersing agent. |
| Applications | W/O emulsifier in cosmetics & topical pharmaceuticals; agricultural emulsifiable concentrates; metalworking fluids; leather processing; corrosion inhibitor formulations |
| Storage | Store in sealed drums away from heat sources. Liquid at room temperature. |

| 57. Sorbitan Tristearate [Non-ionic Emulsifier (Span 65)] |  |
| --- | --- |
| Chemical Formula | C₆₀H₁₁₄O₈ |
| HS Code | 34029090 |
| CAS Number | 26658-19-5 |
| Appearance | Cream to tan waxy solid |
| Odour | Faint fatty odour |
| Density | ~0.98 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | 53–57°C |
| Flash Point | >200°C |
| Solubility | Dispersible in warm water; soluble in oils |
| pH | 5.0–7.5 |
| Purity / Grade | ≥97% |
| Packaging | 25 kg bags |
| Description | Sorbitan Tristearate (Span 65) is a highly lipophilic emulsifier and crystal modifier used to control polymorphism in chocolate and confectionery and as a W/O emulsifier. |
| Applications | Chocolate & confectionery crystal modifier (bloom inhibitor); W/O emulsifier; cosmetic formulations; polishes and coatings |
| Storage | Store in sealed containers below 30°C in dry conditions. |

| 58. VAM-VeoVa (Vinyl Acetate Monomer / Vinyl Ester of Versatic Acid) [Emulsion Polymer Monomer] |  |
| --- | --- |
| Chemical Formula | CH₃COOCH=CH₂ / vinyl ester of C10 acid |
| HS Code | 39089090 ¦ 29212990 |
| CAS Number | 9/3/8002 |
| Appearance | Clear, colourless liquid (VAM); viscous liquid (VeoVa) |
| Odour | Characteristic ester odour (VAM); mild (VeoVa) |
| Density | 0.932 g/cm³ (VAM) |
| Boiling Point | 72.7°C (VAM) |
| Melting Point | -93°C |
| Flash Point | -8°C (VAM) |
| Solubility | Partially soluble in water (2%); miscible with organics |
| pH | N/A |
| Purity / Grade | ≥99.5% |
| Packaging | ISO tanks / drums |
| Description | VAM-VeoVa copolymer emulsions are the basis for exterior architectural coatings, woodworking adhesives, and construction materials. They offer excellent weatherability and adhesion. |
| Applications | Exterior architectural emulsion paints; woodworking adhesives; construction emulsions; cement modifiers; non-woven fabric binders |
| Storage | VAM: highly flammable — store in cool, explosion-proof storage. VeoVa: store in sealed containers away from heat. |

| 59. Poly Vinyl Acetate Homopolymer [Adhesive Polymer Emulsion] |  |
| --- | --- |
| Chemical Formula | (CH₂CH(OOCCH₃))n |
| HS Code | 39069090 |
| CAS Number | 9003-20-7 |
| Appearance | White milky to slightly yellowish emulsion |
| Odour | Mild ester/acetic odour |
| Density | 1.18–1.19 g/cm³ |
| Boiling Point | >100°C |
| Melting Point | N/A (glass transition ~30°C) |
| Flash Point | >100°C |
| Solubility | Emulsion form — dispersible; dried film water-resistant |
| pH | 4.0–7.0 |
| Purity / Grade | ≥50–55% solids (typical emulsion) |
| Packaging | 200 kg drums / IBC |
| Description | Polyvinyl Acetate (PVAc) Homopolymer emulsion is the classic wood adhesive (PVA glue), used in woodworking, paper lamination, bookbinding, and packaging applications. |
| Applications | Woodworking adhesive (PVA glue); paper lamination; bookbinding; packaging adhesives; school/craft glue; construction adhesive |
| Storage | Store at 5–30°C. Protect from freezing and direct sunlight. Keep containers tightly sealed. |

| 60. Pure Acrylic Emulsion [Paint / Coating Binder] |  |
| --- | --- |
| Chemical Formula | Poly(acrylic acid/acrylate ester) copolymer |
| HS Code | 39069090 ¦ 390590 |
| CAS Number | 25068-38-6 |
| Appearance | White milky emulsion |
| Odour | Slight ammonia or acrylic odour |
| Density | 1.02–1.06 g/cm³ |
| Boiling Point | >100°C |
| Melting Point | N/A |
| Flash Point | >100°C |
| Solubility | Emulsion form — water-based; dried film water-resistant |
| pH | 7.5–9.5 |
| Purity / Grade | ≥45–50% solids |
| Packaging | 200 kg drums / IBC |
| Description | Pure Acrylic Emulsions are high-quality binder resins for exterior and interior architectural paints, providing excellent weatherability, UV resistance, gloss retention, and colour stability. |
| Applications | Premium exterior & interior wall paints; roof coatings; wood coatings; elastomeric coatings; primers |
| Storage | Store at 5–35°C. Protect from freezing. Keep sealed — skin formation on exposure. |

| 61. Acrylic Styrene Emulsion [Paint / Coating Binder] |  |
| --- | --- |
| Chemical Formula | Acrylic acid ester / styrene copolymer |
| HS Code | 39069090 |
| CAS Number | 868765-93-9 |
| Appearance | White, milky emulsion |
| Odour | Mild styrenic odour |
| Density | 1.03–1.08 g/cm³ |
| Boiling Point | >100°C |
| Melting Point | N/A (Tg varies by grade) |
| Flash Point | >100°C |
| Solubility | Emulsion form — water-based |
| pH | 7.0–9.5 |
| Purity / Grade | ≥47–50% solids |
| Packaging | 200 kg drums / IBC |
| Description | Styrene-Acrylic copolymer emulsions are cost-effective binders for interior and exterior paints, paper coatings, and adhesives. They offer good hardness, water resistance, and blocking resistance. |
| Applications | Interior & exterior wall emulsion paints; paper coating; construction materials; adhesives; textile finishing |
| Storage | Store at 5–35°C. Do not freeze. Keep sealed. pH sensitive — avoid acidic contamination. |

| 62. Phenalkamine Hardeners (MAXAMINE) [Epoxy Curing Agent / Hardener] |  |
| --- | --- |
| Chemical Formula | Cardanol-based polyamine |
| HS Code | 29212990 |
| CAS Number | 68410-23-1 |
| Appearance | Dark amber to brown viscous liquid |
| Odour | Mild amine odour |
| Density | 0.98–1.05 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | N/A (liquid) |
| Flash Point | >100°C |
| Solubility | Insoluble in water; soluble in organic solvents and epoxy resins |
| pH | N/A |
| Purity / Grade | Amine value: 200–350 mg KOH/g |
| Packaging | 200 L drums / IBC |
| Description | Phenalkamine (Cardanol-based) hardeners are bio-based epoxy curing agents offering excellent moisture tolerance, chemical resistance, and flexibility. They are ideal for marine coatings, concrete protection, and adhesives in humid conditions. |
| Applications | Marine & protective coatings; floor coatings; concrete coatings; adhesives; fast-cure systems; underwater coatings |
| Storage | Store in sealed containers away from moisture and moisture-sensitive epoxy resins. Use PPE. |

| 63. Polyamide Hardeners (MAXAMIDE) [Epoxy Curing Agent / Hardener] |  |
| --- | --- |
| Chemical Formula | Dimer fatty acid polyamide |
| HS Code | 29252990 |
| CAS Number | Blend |
| Appearance | Light yellow to amber viscous liquid |
| Odour | Faint fatty/amine odour |
| Density | 0.95–1.02 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | N/A |
| Flash Point | >100°C |
| Solubility | Insoluble in water; compatible with epoxy resins |
| pH | N/A |
| Purity / Grade | Amine value: 200–400 mg KOH/g |
| Packaging | 200 L drums |
| Description | Polyamide hardeners, derived from dimerized fatty acids and polyamines, are widely used epoxy curing agents known for excellent adhesion, flexibility, and moisture tolerance at ambient temperatures. |
| Applications | General-purpose epoxy coatings; floor coatings; marine coatings; primers; adhesives; potting compounds |
| Storage | Store in sealed containers. Hygroscopic — protect from moisture. Handle with gloves and eye protection. |

| 64. Epoxy Resin (EPOMAX) [Thermoset Resin] |  |
| --- | --- |
| Chemical Formula | Bisphenol A diglycidyl ether (DGEBA) |
| HS Code | 39073010 |
| CAS Number | 25068-38-6 |
| Appearance | Clear to slightly yellow viscous liquid (liquid grades) or solid (solid grades) |
| Odour | Mild epoxy/aromatic odour |
| Density | 1.16–1.20 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | Varies (liquid at RT for standard grades) |
| Flash Point | >200°C |
| Solubility | Insoluble in water; soluble in ketones and esters |
| pH | N/A |
| Purity / Grade | Epoxide equivalent weight: 182–196 g/eq |
| Packaging | 200 L drums / 20 kg cans |
| Description | Liquid Bisphenol A Epoxy Resins are the most widely used thermoset resins, offering outstanding chemical resistance, adhesion, and mechanical strength when cured with polyamine, polyamide, or anhydride hardeners. |
| Applications | Protective & marine coatings; adhesives; electrical encapsulation & PCB laminates; flooring systems; civil engineering composites; automotive coatings |
| Storage | Store in sealed containers below 30°C. Keep away from curing agents until use. Long shelf life if stored correctly. |

| 65. Phenoxyethanol [Preservative / Cosmetic Ingredient] |  |
| --- | --- |
| Chemical Formula | C₈H₁₀O₂ |
| HS Code | 29071990 |
| CAS Number | 122-99-6 |
| Appearance | Colourless, oily liquid |
| Odour | Faint rose-like odour |
| Density | 1.102–1.108 g/cm³ |
| Boiling Point | 245°C |
| Melting Point | 14°C |
| Flash Point | 132°C |
| Solubility | 2.67 g/100 mL (20°C) in water; miscible with ethanol and ether |
| pH | N/A |
| Purity / Grade | ≥99.5% |
| Packaging | 200 kg drums |
| Description | Phenoxyethanol is a broad-spectrum preservative used in cosmetics and personal care products as an alternative to parabens, offering activity against bacteria and yeast at low concentrations (≤1%). |
| Applications | Cosmetic preservative (shampoo, lotion, cream); pharmaceutical preservative; vaccine adjuvant; industrial antimicrobial; perfume fixative |
| Storage | Store in sealed containers at ambient temperature. Avoid prolonged skin contact. |

| 66. Chlorhexidine Gluconate (MAXICIDE) [Broad-spectrum Antimicrobial / Disinfectant] |  |
| --- | --- |
| Chemical Formula | C₂₂H₃₀Cl₂N₁₀·2C₆H₁₂O₇ |
| HS Code | 29332990 |
| CAS Number | 18472-51-0 |
| Appearance | Clear, colourless to pale yellow aqueous solution |
| Odour | Faint characteristic odour |
| Density | 1.06–1.07 g/cm³ (20% solution) |
| Boiling Point | >100°C |
| Melting Point | N/A (solution) |
| Flash Point | Non-flammable |
| Solubility | Soluble in water and ethanol |
| pH | 5.0–7.5 |
| Purity / Grade | ≥20% active chlorhexidine gluconate (standard solution) |
| Packaging | 200 kg drums / IBC |
| Description | Chlorhexidine Gluconate (CHG) is a potent antimicrobial agent active against gram-positive and gram-negative bacteria, yeasts, and some viruses. It is widely used as a surgical scrub, wound antiseptic, and veterinary disinfectant. |
| Applications | Surgical hand scrub & skin antiseptic; wound disinfectant; mouthwash (dental care); veterinary disinfectant; pharmaceutical preservative; food industry hygiene |
| Storage | Store at 15–30°C in sealed containers away from light. Incompatible with anionic surfactants and chlorine-containing compounds. |

| 67. Monochloro Acetic Acid (MCAA) [Chemical Intermediate] |  |
| --- | --- |
| Chemical Formula | ClCH₂COOH |
| HS Code | 29154010 |
| CAS Number | 79-11-8 |
| Appearance | White to off-white crystalline solid (or liquid above 63°C) |
| Odour | Pungent, acrid odour |
| Density | 1.575 g/cm³ |
| Boiling Point | 189°C |
| Melting Point | 63°C |
| Flash Point | 126°C |
| Solubility | Freely soluble in water (85.5 g/100 mL at 25°C) |
| pH | <1 (1% aqueous solution) |
| Purity / Grade | ≥99.5% (solid grade) |
| Packaging | 25 kg bags / drums |
| Description | Monochloroacetic Acid (MCAA) is a highly reactive chemical intermediate used in the synthesis of CMC/SCMC, herbicides (glycine, 2,4-D, MCPA), and surfactants. |
| Applications | SCMC/CMC production (carboxymethylation of cellulose); herbicide synthesis (glyphosate, 2,4-D); surfactant synthesis; dye intermediates; pharmaceutical synthesis |
| Storage | Corrosive — store in sealed containers away from moisture and alkalis. Full PPE required. Very toxic — handle with extreme care. |

| 68. Sodium Monochloro Acetate (SMCA) [Chemical Intermediate] |  |
| --- | --- |
| Chemical Formula | ClCH₂COONa |
| HS Code | 29154010 |
| CAS Number | 3926-62-3 |
| Appearance | White crystalline powder |
| Odour | Odourless to faint |
| Density | 1.76 g/cm³ |
| Boiling Point | Decomposes >250°C |
| Melting Point | ~300°C |
| Flash Point | Non-flammable (solid) |
| Solubility | Freely soluble in water |
| pH | 5.5–7.5 (10% solution) |
| Purity / Grade | ≥98% |
| Packaging | 25 kg bags |
| Description | Sodium Monochloroacetate (SMCA) is the sodium salt of MCAA, used as a safer-handling alternative for MCAA in the synthesis of CMC, thioglycolic acid, and other industrial chemicals. |
| Applications | CMC/SCMC production; herbicide synthesis; thioglycolic acid production; surfactant synthesis; pharmaceutical intermediates |
| Storage | Store in dry, sealed containers. Avoid contact with strong acids, which regenerate free MCAA. Use PPE. |

| 69. Dimethyl Amino Propyl Amine (DMAPA) [Amine / Chemical Intermediate] |  |
| --- | --- |
| Chemical Formula | (CH₃)₂N(CH₂)₃NH₂ |
| HS Code | 29212990 |
| CAS Number | 109-55-7 |
| Appearance | Colourless, mobile liquid |
| Odour | Strong amine odour |
| Density | 0.815–0.820 g/cm³ |
| Boiling Point | 135°C |
| Melting Point | -70°C |
| Flash Point | 35°C |
| Solubility | Miscible with water, alcohols, and most organic solvents |
| pH | >11 (aqueous solution) |
| Purity / Grade | ≥99.0% |
| Packaging | 200 kg drums / ISO tanks |
| Description | DMAPA (3-(Dimethylamino)-1-propylamine) is a key intermediate for the synthesis of CAPB (cocamidopropyl betaine), amphoacetates, and other amphoteric surfactants used in personal care. |
| Applications | CAPB synthesis (betaine surfactants); amphoacetate synthesis; cationic biocides synthesis; epoxy curing agent; textile softener; corrosion inhibitor |
| Storage | Flammable and corrosive — store in sealed containers away from heat and oxidizers. Strong amine — full PPE required. |

| 70. Benzyl Alcohol [Solvent / Preservative / Fragrance Ingredient] |  |
| --- | --- |
| Chemical Formula | C₆H₅CH₂OH |
| HS Code | 29062100 |
| CAS Number | 100-51-6 |
| Appearance | Clear, colourless liquid |
| Odour | Faint pleasant aromatic odour |
| Density | 1.041–1.045 g/cm³ |
| Boiling Point | 205°C |
| Melting Point | -15°C |
| Flash Point | 93°C |
| Solubility | 4% v/v in water; miscible with alcohols and organic solvents |
| pH | N/A |
| Purity / Grade | ≥99.5% |
| Packaging | 200 kg drums |
| Description | Benzyl Alcohol is a versatile aromatic solvent and fragrance ingredient used as a preservative in cosmetics and pharmaceuticals, an ink and resin solvent, and in flavour/fragrance synthesis. |
| Applications | Cosmetic preservative (paraben alternative); pharmaceutical solvent/preservative; fragrance ingredient; paint & varnish solvent; ink & resin solvent; plasticizer intermediate |
| Storage | Store in sealed containers at ambient temperature. Combustible — keep away from ignition sources. |

| 71. Benzyl Chloride [Chemical Intermediate / Chlorinating Agent] |  |
| --- | --- |
| Chemical Formula | C₆H₅CH₂Cl |
| HS Code | 29039940 |
| CAS Number | 100-44-7 |
| Appearance | Colourless, fuming liquid |
| Odour | Very pungent, irritating |
| Density | 1.100 g/cm³ |
| Boiling Point | 179°C |
| Melting Point | -39°C |
| Flash Point | 67°C |
| Solubility | Insoluble in water; miscible with organic solvents |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 200 kg drums (stainless steel or HDPE lined) |
| Description | Benzyl Chloride is an important chemical intermediate for the synthesis of benzyl compounds including benzyl alcohol, benzaldehyde, benzalkonium chloride, and quaternary ammonium compounds. |
| Applications | Benzyl compound synthesis; BKC/QAC synthesis; perfume & fragrance intermediates; pharmaceutical synthesis; dye intermediates; resin modifier |
| Storage | Lachrymatory — highly irritating vapour. Store in sealed containers in cool, ventilated area with corrosion-resistant lining. Full PPE essential. |

| 72. Castor Oil Ethoxylates [Non-ionic Emulsifier / Solubilizer] |  |
| --- | --- |
| Chemical Formula | Castor oil-(EO)n |
| HS Code | 34029090 |
| CAS Number | 61791-12-6 |
| Appearance | Clear to slightly yellow viscous liquid |
| Odour | Faint fatty odour |
| Density | 1.00–1.06 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | N/A |
| Flash Point | >200°C |
| Solubility | Soluble/dispersible in water (higher EO content more hydrophilic) |
| pH | 6.0–8.0 |
| Purity / Grade | ≥99% ester content |
| Packaging | 200 kg drums |
| Description | Castor Oil Ethoxylates are non-ionic surfactants derived from the ethoxylation of castor oil. They function as emulsifiers, solubilizers, and wetting agents, and are widely used to formulate fragrances and essential oils into aqueous systems. |
| Applications | Fragrance solubilizer in personal care; textile lubricants & softeners; metalworking emulsifiers; agricultural adjuvants; pigment dispersants; cosmetic emulsifiers |
| Storage | Store in sealed containers at ambient temperature. Avoid freezing. Keep away from strong acids. |

| 73. Nonyl Phenol Ethoxylates (NPE) [Non-ionic Surfactant (Industrial)] |  |
| --- | --- |
| Chemical Formula | C₉H₁₉C₆H₄(OCH₂CH₂)nOH |
| HS Code | 34029090 |
| CAS Number | 9016-45-9 |
| Appearance | Clear to slightly yellow liquid |
| Odour | Mild characteristic odour |
| Density | 1.02–1.08 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | N/A |
| Flash Point | >200°C |
| Solubility | Soluble/dispersible in water; soluble in organics |
| pH | 6.0–8.0 |
| Purity / Grade | ≥99% |
| Packaging | 200 kg drums |
| Description | Nonyl Phenol Ethoxylates (NPE) are industrial non-ionic surfactants used as emulsifiers, wetting agents, and detergents in textile, metal, and industrial cleaning applications. Note: restricted for consumer use in EU and many markets due to environmental concerns. |
| Applications | Industrial textile processing; metalworking emulsifiers; industrial cleaning; agrochemical emulsifiers; emulsion polymerization |
| Storage | Store in sealed drums at ambient temperature. Restricted for certain consumer applications — check local regulations. |

| 74. Polysorbate 80 (Tween 80) [Non-ionic Emulsifier (O/W)] |  |
| --- | --- |
| Chemical Formula | POE(20) sorbitan monooleate |
| HS Code | 34029090 |
| CAS Number | 9005-65-6 |
| Appearance | Amber, oily liquid |
| Odour | Faint fatty/oily odour |
| Density | 1.06–1.09 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | <0°C |
| Flash Point | >200°C |
| Solubility | Miscible with water |
| pH | 5.0–7.0 (5% aqueous solution) |
| Purity / Grade | ≥99% (HLB 15) |
| Packaging | 200 kg drums |
| Description | Polysorbate 80 (Tween 80) is a highly hydrophilic non-ionic emulsifier/solubilizer used in food, pharmaceuticals, and cosmetics to produce stable O/W emulsions, solubilize oils and vitamins, and improve bioavailability. |
| Applications | Food O/W emulsifier (E433); pharmaceutical drug solubilizer & vaccine adjuvant; cosmetic emulsifier; ice cream stabilizer; vitamin E solubilizer |
| Storage | Store in sealed containers at ambient temperature away from light. Avoid high temperatures. |

| 75. Polysorbate 60 (Tween 60) [Non-ionic Emulsifier (O/W)] |  |
| --- | --- |
| Chemical Formula | POE(20) sorbitan monostearate |
| HS Code | 34029090 |
| CAS Number | 9005-67-8 |
| Appearance | Cream to amber soft solid |
| Odour | Faint fatty odour |
| Density | ~1.10 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | 25–28°C |
| Flash Point | >200°C |
| Solubility | Dispersible/miscible with water |
| pH | 5.0–7.0 |
| Purity / Grade | ≥99% (HLB 14.9) |
| Packaging | 200 kg drums |
| Description | Polysorbate 60 (Tween 60) is a food-grade hydrophilic emulsifier used in baked goods, dairy products, and confectionery to improve texture, volume, and shelf life. |
| Applications | Food emulsifier (E435) in bakery, cakes, desserts; W/O and O/W emulsions; cosmetic cream emulsifier; pharmaceutical emulsifier |
| Storage | Store at room temperature away from moisture. May solidify at low temperatures — melt gently before use. |

| 76. Polysorbate 20 (Tween 20) [Non-ionic Emulsifier / Solubilizer] |  |
| --- | --- |
| Chemical Formula | POE(20) sorbitan monolaurate |
| HS Code | 34029090 |
| CAS Number | 9005-64-5 |
| Appearance | Yellow, oily liquid |
| Odour | Faint fatty odour |
| Density | 1.09–1.10 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | <0°C |
| Flash Point | >200°C |
| Solubility | Miscible with water (HLB 16.7) |
| pH | 5.0–7.0 |
| Purity / Grade | ≥99% |
| Packaging | 200 kg drums |
| Description | Polysorbate 20 (Tween 20) is the most hydrophilic of the Tween series, used as a solubilizer for fragrance oils, essential oils, and vitamins in aqueous formulations, and as a wetting agent in diagnostics. |
| Applications | Fragrance/essential oil solubilizer; cosmetic toner & mist formulations; pharmaceutical solubilizer; laboratory reagent (ELISA, protein stabilizer); food emulsifier (E432) |
| Storage | Store in sealed containers at ambient temperature. Very hygroscopic — keep sealed. |

| 77. Acid Thickener (Carbopol-type / HASE thickener) [Rheology Modifier / Thickener] |  |
| --- | --- |
| Chemical Formula | Crosslinked polyacrylic acid / Hydrophobically modified acrylic copolymer |
| HS Code | 39069090 |
| CAS Number | 9003-01-4 / Blend |
| Appearance | White, fine powder (dry); clear gel on neutralization |
| Odour | Odourless |
| Density | ~1.4 g/cm³ (powder) |
| Boiling Point | N/A |
| Melting Point | N/A |
| Flash Point | Non-flammable (powder) |
| Solubility | Swells in water; fully gels on neutralization with alkali |
| pH | 3.0–4.0 (1% aqueous suspension); 6–7 after neutralization |
| Purity / Grade | Per specification |
| Packaging | 25 kg bags |
| Description | Acid Thickeners (crosslinked polyacrylic acid polymers) are rheology modifiers that form highly efficient thickeners for surfactant systems, cosmetics, and household cleaners after neutralization. |
| Applications | Shampoo and shower gel thickener; liquid detergent viscosifier; hand sanitizer gel base; cosmetic gel/lotion thickener; industrial cleaner thickener |
| Storage | Store in sealed bags away from moisture. Powder is a mild dust hazard — use respiratory protection. |

| 78. Piroctone Olamine [Anti-Dandruff Active / Biocide] |  |
| --- | --- |
| Chemical Formula | C₁₆H₂₈N₂O₃ |
| HS Code | 29332990 |
| CAS Number | 68890-66-4 |
| Appearance | White to off-white crystalline powder |
| Odour | Odourless to faint |
| Density | ~1.15 g/cm³ |
| Boiling Point | N/A |
| Melting Point | 170–175°C |
| Flash Point | Non-flammable |
| Solubility | 1% in water; freely soluble in ethanol and polar solvents |
| pH | 8.0–11.0 (1% aqueous solution) |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags |
| Description | Piroctone Olamine (Octopirox) is a broad-spectrum antifungal agent and anti-dandruff active effective against Malassezia furfur. It is milder than zinc pyrithione and compatible with most cosmetic formulations. |
| Applications | Anti-dandruff shampoos; anti-fungal cosmetics; scalp treatments; face wash (acne treatment); textile antimicrobial finish |
| Storage | Store in sealed containers in a dry, cool place away from light. Stable under recommended storage conditions. |

| 79. Triclosan [Antimicrobial / Preservative] |  |
| --- | --- |
| Chemical Formula | C₁₂H₇Cl₃O₂ |
| HS Code | 29081900 |
| CAS Number | 3380-34-5 |
| Appearance | White to off-white crystalline powder |
| Odour | Faint phenolic odour |
| Density | 1.55 g/cm³ |
| Boiling Point | 280–290°C |
| Melting Point | 54–57°C |
| Flash Point | >100°C |
| Solubility | Very slightly soluble in water (10 mg/L); soluble in ethanol, propylene glycol |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags |
| Description | Triclosan is a broad-spectrum antibacterial and antifungal compound used in personal care products, industrial antimicrobials, and textiles. Note: usage in rinse-off consumer products is restricted in many markets. |
| Applications | Antibacterial soap & hand wash (industrial/regulated markets); toothpaste (antiplaque); medical/surgical hand scrub; textile antimicrobial treatment; industrial disinfection |
| Storage | Store in sealed containers away from light and heat. Follow local regulations for use and disposal. |

| 80. Propyl Paraben [Cosmetic / Pharmaceutical Preservative] |  |
| --- | --- |
| Chemical Formula | C₁₀H₁₂O₃ |
| HS Code | 29163990 |
| CAS Number | 94-13-3 |
| Appearance | White crystalline powder |
| Odour | Odourless to faint |
| Density | 1.234 g/cm³ |
| Boiling Point | 295°C |
| Melting Point | 95–98°C |
| Flash Point | >100°C |
| Solubility | 0.05 g/100 mL (water); freely soluble in ethanol |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags |
| Description | Propyl Paraben (propyl 4-hydroxybenzoate) is a widely used antimicrobial preservative in cosmetics, pharmaceuticals, and food (E216), effective against moulds and yeasts at low concentrations. |
| Applications | Cosmetic preservative (creams, lotions, shampoos); pharmaceutical preservative (oral & topical); food preservative (E216); personal care formulations |
| Storage | Store in sealed containers in a dry, cool place. Stable under normal conditions. |

| 81. PROMIX (Methylisothiazolinone / Methylchloroisothiazolinone blend) [Biocide / Preservative] |  |
| --- | --- |
| Chemical Formula | MIT/CMIT blend (5-chloro-2-methyl-4-isothiazolin-3-one / 2-methyl-4-isothiazolin-3-one) |
| HS Code | 29332990 |
| CAS Number | Mixture |
| Appearance | Clear to light amber aqueous solution |
| Odour | Slight characteristic odour |
| Density | ~1.02 g/cm³ |
| Boiling Point | >100°C |
| Melting Point | N/A |
| Flash Point | Non-flammable |
| Solubility | Fully miscible with water |
| pH | 2.0–5.0 |
| Purity / Grade | 1.5–2.0% active (standard solution) |
| Packaging | 25 kg / 200 kg drums |
| Description | PROMIX (CMIT/MIT blend) is a highly effective broad-spectrum biocide preservative used in industrial water treatment, paints, metalworking fluids, personal care, and household products. |
| Applications | Industrial water treatment biocide; paint & coating preservative (in-can); metalworking fluid preservative; adhesive & sealant preservative; cosmetic preservative |
| Storage | Store in sealed containers at ambient temperature. Oxidizing agent — keep away from combustibles. Full PPE required. |

| 82. Polyethylene Glycol (PEG) [Humectant / Solubilizer / Industrial Lubricant] |  |
| --- | --- |
| Chemical Formula | H(OCH₂CH₂)nOH |
| HS Code | 39072090 |
| CAS Number | 25322-68-3 |
| Appearance | Clear liquid (low MW) to white waxy solid (high MW) |
| Odour | Odourless |
| Density | 1.11–1.22 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | 4–66°C (varies by grade) |
| Flash Point | >250°C |
| Solubility | Freely miscible with water |
| pH | 4.5–7.5 |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags (solid) / 200 kg drums (liquid) |
| Description | Polyethylene Glycol (PEG) is a versatile polyether alcohol available in a wide range of molecular weights (PEG 200 to PEG 8000), used as a humectant, solubilizer, lubricant, and binder across cosmetics, pharma, food, and industry. |
| Applications | Cosmetic humectant & emollient; pharmaceutical excipient; food additive (E1521); metalworking lubricant; paper softener; chemical intermediate (surfactant synthesis) |
| Storage | Store in sealed containers at ambient temperature. Higher MW grades: protect from moisture. |

| 83. Para Chloro Meta Xylenol (PCMX) [Antiseptic / Antimicrobial] |  |
| --- | --- |
| Chemical Formula | C₈H₉ClO |
| HS Code | 29081900 |
| CAS Number | 88-04-0 |
| Appearance | White to off-white crystalline powder |
| Odour | Characteristic phenolic/antiseptic odour |
| Density | 1.24 g/cm³ |
| Boiling Point | 246°C |
| Melting Point | 115–116°C |
| Flash Point | >110°C |
| Solubility | 0.3 g/100 mL (water); freely soluble in ethanol and propylene glycol |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags |
| Description | Para Chloro Meta Xylenol (PCMX / Chloroxylenol) is an antimicrobial phenolic compound used in disinfectant formulations, antiseptic skin care, veterinary products, and as a preservative in cosmetics. |
| Applications | Antiseptic hand wash & wound care; household disinfectants & liquid antiseptic; veterinary formulations; industrial disinfection; cosmetic preservative |
| Storage | Store in sealed containers away from light and heat. Mild irritant — use PPE. |

| 84. Octyl Methoxy Cinnamate (OMC) [UV Filter / Sunscreen Active] |  |
| --- | --- |
| Chemical Formula | C₁₈H₂₆O₃ |
| HS Code | 29161990 |
| CAS Number | 5466-77-3 |
| Appearance | Clear, pale yellow to yellow oily liquid |
| Odour | Faint floral/sweet odour |
| Density | 1.009–1.015 g/cm³ |
| Boiling Point | 198°C (at 1 mmHg) |
| Melting Point | -25°C |
| Flash Point | >100°C |
| Solubility | Insoluble in water; soluble in oils and organic solvents |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 200 kg drums |
| Description | Octyl Methoxy Cinnamate (Ethylhexyl Methoxycinnamate) is the most widely used UVB sunscreen filter (absorbs 280–320 nm), incorporated in sunscreen lotions, day creams, and UV-protective cosmetics. |
| Applications | Sunscreen (SPF contributor); UV-protective day creams & moisturizers; hair UV protection products; lip balms; combined with UVA filters in broad-spectrum sunscreens |
| Storage | Store in sealed drums away from light. May absorb UV — protect from prolonged light exposure during storage. |

| 85. Methyl Paraben [Cosmetic / Pharmaceutical Preservative] |  |
| --- | --- |
| Chemical Formula | C₈H₈O₃ |
| HS Code | 29163990 |
| CAS Number | 99-76-3 |
| Appearance | White crystalline powder |
| Odour | Faint characteristic odour |
| Density | 1.352 g/cm³ |
| Boiling Point | 275°C |
| Melting Point | 125–128°C |
| Flash Point | >100°C |
| Solubility | 0.25 g/100 mL (water); freely soluble in ethanol |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags |
| Description | Methyl Paraben (methyl 4-hydroxybenzoate) is the most widely used paraben preservative in cosmetics, pharmaceuticals, and food (E218), providing effective antibacterial and antifungal protection at low use levels. |
| Applications | Cosmetic preservative; pharmaceutical preservative; food preservative (E218); personal care and household product preservation |
| Storage | Store in sealed bags in cool, dry place. Stable under normal storage conditions. |

| 86. Lauryl Amine Oxide (LAO) [Amphoteric Surfactant / Foam Booster] |  |
| --- | --- |
| Chemical Formula | C₁₂H₂₇NO |
| HS Code | 34024900 |
| CAS Number | 1643-20-5 |
| Appearance | Clear to slightly yellow liquid (30% solution) |
| Odour | Faint fatty/amine odour |
| Density | 0.98–1.02 g/cm³ |
| Boiling Point | >100°C (solution) |
| Melting Point | N/A |
| Flash Point | Non-flammable |
| Solubility | Fully miscible with water |
| pH | 6.0–8.0 |
| Purity / Grade | ≥30% active matter |
| Packaging | 200 kg drums |
| Description | Lauryl Amine Oxide (LAO) is an amphoteric surfactant that functions as an efficient foam booster and viscosity builder in combination with anionic surfactants. It is mild, biodegradable, and compatible with most surfactant systems. |
| Applications | Shampoo & dishwash foam booster; liquid detergent viscosity builder; household & institutional cleaning; mild baby product formulations; foam fire-fighting concentrates |
| Storage | Store in sealed containers at 10–35°C. Avoid freezing. |

| 87. Glyceryl Monostearate (GMS) [Food Emulsifier / Cosmetic Emulsifier] |  |
| --- | --- |
| Chemical Formula | C₂₁H₄₂O₄ |
| HS Code | 29157090 |
| CAS Number | 123-94-4 |
| Appearance | White to cream waxy solid |
| Odour | Faint fatty odour |
| Density | 0.970 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | 56–63°C (pure GMS) |
| Flash Point | >200°C |
| Solubility | Insoluble in water; dispersible in hot water; soluble in oils |
| pH | N/A |
| Purity / Grade | ≥40% monoglyceride (SE grade); ≥90% monoglyceride (distilled/DMG grade) |
| Packaging | 25 kg bags |
| Description | Glyceryl Monostearate (GMS) is a versatile food emulsifier (E471) and cosmetic self-emulsifying wax. It improves texture, shelf life, and emulsification in bakery, dairy, personal care, and pharmaceutical products. |
| Applications | Food emulsifier (E471) in bakery, margarine, ice cream; cosmetic O/W emulsifier (self-emulsifying grade); pharmaceutical excipient; PVC lubricant & anti-static agent |
| Storage | Store in cool, dry, sealed containers below 30°C. Protect from moisture and contamination. |

| 88. Enzymes (Industrial) [Biological Catalyst / Detergent Additive] |  |
| --- | --- |
| Chemical Formula | Protease, Amylase, Lipase, Cellulase (protein-based biocatalysts) |
| HS Code | 35079090 |
| CAS Number | Various |
| Appearance | Beige to brown granular powder or liquid |
| Odour | Faint fermentation odour |
| Density | ~0.5–1.1 g/cm³ |
| Boiling Point | N/A (denaturation above 70°C typically) |
| Melting Point | N/A |
| Flash Point | Non-flammable |
| Solubility | Soluble/dispersible in water |
| pH | 7.0–9.0 (optimum activity, varies by enzyme) |
| Purity / Grade | Activity units per gram/mL (per specification) |
| Packaging | 25 kg bags (solid) / 200 L drums (liquid) |
| Description | Industrial Enzymes (proteases, amylases, lipases, cellulases) are biological catalysts used in detergent formulations to break down stains (protein, starch, fat, cellulose), in food processing, textile, and pharmaceutical applications. |
| Applications | Laundry & dish detergent stain removal; food & beverage processing; textile desizing; leather processing; pharmaceutical manufacturing; biofuel production |
| Storage | Store at 0–10°C for liquid enzymes; cool/dry for powder grades. Avoid heat and UV exposure. Short shelf life — use before expiry. |

| 89. DMDM Hydantoin [Formaldehyde-releasing Cosmetic Preservative] |  |
| --- | --- |
| Chemical Formula | C₇H₁₂N₂O₄ |
| HS Code | 29332990 |
| CAS Number | 6440-58-0 |
| Appearance | Clear, colourless to pale yellow liquid |
| Odour | Faint formaldehyde odour |
| Density | 1.22–1.26 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | N/A |
| Flash Point | Non-flammable |
| Solubility | Fully miscible with water |
| pH | 4.5–6.5 |
| Purity / Grade | ≥55% active in 1,3-dihydroxymethyl-5,5-dimethylhydantoin |
| Packaging | 25 kg / 200 kg drums |
| Description | DMDM Hydantoin is a formaldehyde-releasing preservative used in personal care products to control microbial growth. It slowly releases formaldehyde to inhibit bacteria and fungi. Use requires cautious formulation and compliance with local regulations. |
| Applications | Cosmetic preservative (shampoo, conditioner, lotion); hair care products; industrial water treatment biocide |
| Storage | Store in sealed containers at ambient temperature. Contains formaldehyde — follow SDS and regulatory guidelines. |

| 90. Cetyl Trimethyl Ammonium Chloride (CTAC) [Cationic Surfactant / Hair Conditioner] |  |
| --- | --- |
| Chemical Formula | [CH₃(CH₂)₁₅N(CH₃)₃]Cl |
| HS Code | 29239000 |
| CAS Number | 112-02-7 |
| Appearance | White paste or viscous liquid (30% solution) |
| Odour | Faint fatty/amine odour |
| Density | 0.98–1.02 g/cm³ |
| Boiling Point | >100°C |
| Melting Point | 38°C (solid grade) |
| Flash Point | Non-flammable (solution) |
| Solubility | Soluble in water and ethanol |
| pH | 5.0–8.0 |
| Purity / Grade | ≥30% active matter (solution) |
| Packaging | 200 kg drums |
| Description | Cetyl Trimethyl Ammonium Chloride (CTAC) is a cationic quaternary ammonium compound used as a primary hair conditioning agent, fabric softener, and biocide. |
| Applications | Hair conditioner & rinse (primary conditioning agent); fabric softener; anti-static agent; hair dye vehicle; pharmaceutical excipient; biocide/disinfectant |
| Storage | Store at 15–30°C. Avoid freezing. Keep away from anionic surfactants to prevent incompatibility. |

| 91. Cocodiethanolamide (CDEA) [Non-ionic Foam Booster / Viscosifier] |  |
| --- | --- |
| Chemical Formula | Coconut fatty acid diethanolamide |
| HS Code | 34029090 |
| CAS Number | 61791-31-9 |
| Appearance | Yellow to amber viscous liquid |
| Odour | Faint fatty/soapy odour |
| Density | 0.988–1.005 g/cm³ |
| Boiling Point | >200°C |
| Melting Point | <10°C |
| Flash Point | >200°C |
| Solubility | Dispersible/miscible in water; soluble in surfactant systems |
| pH | 9.0–11.0 |
| Purity / Grade | ≥85% active matter (1:1 grade) |
| Packaging | 200 kg drums |
| Description | Cocodiethanolamide (CDEA / Cocamide DEA) is a non-ionic surfactant derived from coconut oil and diethanolamine, used as a foam booster, viscosity builder, and emulsifier in shampoos, hand soaps, and dishwashing liquids. |
| Applications | Shampoo foam booster & thickener; liquid soap & dishwash formulations; hand wash viscosifier; household & industrial cleaners; corrosion inhibitor in metalworking fluids |
| Storage | Store in sealed containers at 15–35°C. Avoid freezing. Keep away from strong acids. |

| 92. Carbomer (Carbopol) [Rheology Modifier / Gelling Agent] |  |
| --- | --- |
| Chemical Formula | Crosslinked polyacrylic acid polymer |
| HS Code | 39069090 |
| CAS Number | 9003-01-4 |
| Appearance | White, fluffy, light powder |
| Odour | Odourless |
| Density | ~1.41 g/cm³ (true); ~0.2 g/cm³ (bulk) |
| Boiling Point | N/A |
| Melting Point | N/A |
| Flash Point | Non-flammable |
| Solubility | Swells in water; forms clear gel on neutralization with alkali |
| pH | 2.5–3.5 (0.5% aqueous dispersion); 6.0–8.0 (after neutralization) |
| Purity / Grade | Active polymer content per grade specification |
| Packaging | 25 kg bags |
| Description | Carbomers (Carbopol) are high molecular weight crosslinked polyacrylic acid polymers that swell and thicken dramatically in water upon neutralization with alkali, forming clear, stable gels used across cosmetics, pharmaceuticals, and personal care. |
| Applications | Cosmetic gel base (hand sanitizer, hair gel, face gel); pharmaceutical topical gel formulations; eye care gels; oral care; suspension stabilizer in HPC systems |
| Storage | Store in sealed bags in dry conditions. Fine powder — avoid dust inhalation. Neutralize before use with NaOH, TEA, or KOH. |

| 93. Benzophenone-4 (BP-4) [UV Filter / Sunscreen Active] |  |
| --- | --- |
| Chemical Formula | C₁₄H₁₂O₆S |
| HS Code | 29145000 |
| CAS Number | 4065-45-6 |
| Appearance | Cream to off-white powder |
| Odour | Odourless to faint |
| Density | ~1.4 g/cm³ |
| Boiling Point | N/A |
| Melting Point | 199–202°C |
| Flash Point | Non-flammable (solid) |
| Solubility | Slightly soluble in water; soluble in ethanol and propylene glycol |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags |
| Description | Benzophenone-4 (Sulisobenzone) is a water-soluble UVA+UVB sunscreen filter used in cosmetic and personal care products to protect against UV radiation. It is particularly useful in water-based formulations where oil-soluble UV filters are difficult to incorporate. |
| Applications | Sunscreen formulations; hair UV protection sprays; UV-protective skin care; combined UV protection systems; photostabilizer for other UV actives |
| Storage | Store in sealed bags in cool, dry place away from light. |

| 94. Benzophenone-3 (Oxybenzone) [UV Filter / Sunscreen Active (UVA+UVB)] |  |
| --- | --- |
| Chemical Formula | C₁₄H₁₂O₃ |
| HS Code | 29145000 |
| CAS Number | 131-57-7 |
| Appearance | White to pale yellow crystalline powder |
| Odour | Faint characteristic odour |
| Density | 1.217 g/cm³ |
| Boiling Point | 313°C |
| Melting Point | 62–63°C |
| Flash Point | >100°C |
| Solubility | Insoluble in water; soluble in oils and ethanol |
| pH | N/A |
| Purity / Grade | ≥99.0% |
| Packaging | 25 kg bags |
| Description | Benzophenone-3 (Oxybenzone) is a widely used organic sunscreen ingredient that absorbs both UVA (320–400 nm) and UVB (280–320 nm) radiation. Restricted in some markets due to potential endocrine disruption concerns. |
| Applications | Sunscreen lotions & creams; SPF moisturizers; lip balms with UV protection; hair sunscreen sprays; combined UV filter systems |
| Storage | Store in sealed bags in cool, dry place away from heat and light. |

| 95. Benzalkonium Chloride (BKC) [Cationic Biocide / Disinfectant / Preservative] |  |
| --- | --- |
| Chemical Formula | [C₆H₅CH₂N(CH₃)₂R]Cl (R = C8-C18 alkyl) |
| HS Code | 29239000 |
| CAS Number | 8001-54-5 |
| Appearance | White to off-white powder or colourless liquid (50% solution) |
| Odour | Faint characteristic amine odour |
| Density | 0.98–1.02 g/cm³ (50% solution) |
| Boiling Point | >100°C |
| Melting Point | ~40°C (solid grade) |
| Flash Point | Non-flammable (solution) |
| Solubility | Freely soluble in water and ethanol |
| pH | 5.0–8.0 (10% solution) |
| Purity / Grade | ≥50% active (liquid); ≥97% (powder) |
| Packaging | 200 kg drums (liquid) / 25 kg bags (powder) |
| Description | Benzalkonium Chloride (BKC) is a cationic quaternary ammonium biocide with broad-spectrum antimicrobial activity against bacteria, fungi, algae, and some viruses. It is used as a disinfectant, preservative, and antiseptic across multiple industries. |
| Applications | Disinfectant hand rubs & hospital antiseptic; swimming pool algicide; cosmetic/personal care preservative; food-contact surface sanitizer; water treatment biocide; nasal drops preservative |
| Storage | Store in sealed containers at ambient temperature. Incompatible with anionic surfactants. Corrosive at high concentrations. |

| 96. Butylated Hydroxytoluene (BHT) [Antioxidant / Preservative] |  |
| --- | --- |
| Chemical Formula | C₁₅H₂₄O |
| HS Code | 29071990 |
| CAS Number | 128-37-0 |
| Appearance | White crystalline solid |
| Odour | Faint characteristic phenolic odour |
| Density | 1.048 g/cm³ |
| Boiling Point | 265°C |
| Melting Point | 69–72°C |
| Flash Point | 127°C |
| Solubility | Insoluble in water; soluble in oils, ethanol, and acetone |
| pH | N/A |
| Purity / Grade | ≥99.5% |
| Packaging | 25 kg bags |
| Description | Butylated Hydroxytoluene (BHT) is a phenolic antioxidant used to prevent oxidative rancidity in food, cosmetics, petroleum products, and rubber. It is one of the most widely used synthetic antioxidants globally. |
| Applications | Food antioxidant (E321) in fats/oils, snack foods; cosmetic antioxidant (in lipsticks, creams); petroleum & lubricant stabilizer; rubber antioxidant; packaging (migration into food products) |
| Storage | Store in sealed containers away from heat and oxidizing agents. Combustible solid — avoid ignition sources. |

| 97. Calcium Hypochlorite [Disinfectant / Sanitizer / Bleaching Agent] |  |
| --- | --- |
| Chemical Formula | Ca(OCl)₂ |
| HS Code | 28281090 |
| CAS Number | 7778-54-3 |
| Appearance | White granular powder or tablets |
| Odour | Strong chlorine/bleach odour |
| Density | 2.35 g/cm³ |
| Boiling Point | Decomposes |
| Melting Point | 100°C (decomposes) |
| Flash Point | Non-flammable (strong oxidizer) |
| Solubility | 21 g/100 mL (20°C) in water |
| pH | 10–12 (aqueous solution) |
| Purity / Grade | 65–70% available chlorine (granular) |
| Packaging | 45 kg drums (granular) / 45 kg drums (tablets) |
| Description | Calcium Hypochlorite is a powerful chlorine-based disinfectant, bleaching agent, and sanitizer used in drinking water treatment, swimming pools, food industry sanitation, and household bleach preparation. |
| Applications | Swimming pool disinfection; drinking water treatment; food industry CIP sanitation; hospital/institutional disinfection; laundry bleaching; algae control in irrigation |
| Storage | Strong oxidizer — store away from flammables, organics, and reducing agents. Store in cool, dry, ventilated, acid-free conditions. Avoid moisture. |

| 98. Formaldehyde [Chemical Intermediate / Biocide / Fixative] |  |
| --- | --- |
| Chemical Formula | HCHO (CH₂O) |
| HS Code | 29121100 |
| CAS Number | 50-00-0 |
| Appearance | Clear, colourless liquid (37–40% aqueous solution / formalin) |
| Odour | Pungent, suffocating |
| Density | 1.083–1.090 g/cm³ (37% solution) |
| Boiling Point | -19°C (pure); ~100°C (formalin) |
| Melting Point | -92°C |
| Flash Point | 85°C (37% solution) |
| Solubility | Infinitely miscible with water |
| pH | 3.0–4.5 |
| Purity / Grade | 37–40% formaldehyde (formalin); ≥99.5% (anhydrous) |
| Packaging | 200 L drums / IBC |
| Description | Formaldehyde (formalin solution) is a fundamental chemical intermediate and biocide used in resin synthesis (urea-formaldehyde, phenol-formaldehyde), tissue fixation, leather tanning, and as an industrial disinfectant. |
| Applications | Urea/phenol formaldehyde resin production; tissue fixative/embalming; leather & textile finishing; industrial biocide/disinfectant; explosive & pharmaceuticals intermediate |
| Storage | Store in sealed containers at 15–25°C away from heat, acids, alkalis. Highly toxic and flammable vapour — full PPE essential. Regulatory controls apply. |

| 99. Bleaching Earth (Activated Clay / Fuller's Earth) [Adsorbent / Bleaching Agent] |  |
| --- | --- |
| Chemical Formula | Al₂O₃·SiO₂·MgO (hydrated aluminosilicate) |
| HS Code | 38029019 |
| CAS Number | 14808-60-7 |
| Appearance | Off-white to light tan fine powder |
| Odour | Odourless or earthy |
| Density | ~2.5 g/cm³ (true); ~0.5–0.7 g/cm³ (bulk) |
| Boiling Point | N/A |
| Melting Point | >1000°C |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water |
| pH | 4.0–8.0 (aqueous slurry) |
| Purity / Grade | Bleaching activity per specification (oil decolorization capacity) |
| Packaging | 25 kg bags / bulk |
| Description | Bleaching Earth (activated montmorillonite clay) is used to decolorize and purify vegetable oils, mineral oils, and animal fats by adsorbing pigments, peroxides, soaps, and heavy metals. It is essential in edible oil refining. |
| Applications | Edible oil refining & decolorization (palm, soy, sunflower oils); mineral oil bleaching; biodiesel production; wax bleaching; water treatment (dye adsorption) |
| Storage | Store in sealed bags in dry, cool conditions. Avoid moisture — degrades bleaching capacity. Non-hazardous under normal handling. |

| 100. Phosphoric Acid [Inorganic Acid / Food Additive] |  |
| --- | --- |
| Chemical Formula | H₃PO₄ |
| HS Code | 28092010 |
| CAS Number | 7664-38-2 |
| Appearance | Clear, colourless to slightly yellow viscous liquid or crystalline solid |
| Odour | Odourless |
| Density | 1.685 g/cm³ (85%) |
| Boiling Point | 158°C (85%) |
| Melting Point | 42.4°C (pure) |
| Flash Point | Non-flammable |
| Solubility | Miscible with water in all proportions |
| pH | <1 (concentrated) |
| Purity / Grade | ≥85% (technical/food grade) |
| Packaging | 35 kg carboys / 250 kg HDPE drums / ISO tanks |
| Description | Phosphoric Acid is a moderate-strength mineral acid widely used as an acidity regulator in food & beverages (E338), metal treatment (rust conversion), fertilizer production, and as a chemical intermediate for phosphates. |
| Applications | Food & beverage acidulant (E338) — cola drinks; metal rust treatment & phosphatizing; fertilizer (triple superphosphate) production; pharmaceutical excipient; dental etchant |
| Storage | Store in acid-resistant sealed containers. Corrosive — avoid skin/eye contact. Keep away from alkalis and strong oxidizers. |

| 101. Maize Starch [Food Ingredient / Industrial Binder] |  |
| --- | --- |
| Chemical Formula | (C₆H₁₀O₅)n |
| HS Code | 11081200 |
| CAS Number | 9005-25-8 |
| Appearance | White, fine, free-flowing powder |
| Odour | Faint grain/corn odour |
| Density | ~1.5 g/cm³ (true); ~0.5–0.6 g/cm³ (bulk) |
| Boiling Point | N/A |
| Melting Point | N/A (gelatinizes in water at 62–72°C) |
| Flash Point | Non-flammable |
| Solubility | Insoluble in cold water; gelatinizes in hot water |
| pH | 5.0–7.5 (10% aqueous slurry) |
| Purity / Grade | ≥99% starch (dry basis); moisture ≤14% |
| Packaging | 25 kg bags |
| Description | Maize (Corn) Starch is a natural carbohydrate polymer used as a food thickener, industrial binder, and paper coating agent. It is also a key feedstock for glucose, dextrose, and bioethanol production. |
| Applications | Food thickener & texturizer; paper & board surface sizing; textile sizing; pharmaceutical binder/filler; adhesives; feedstock for glucose/dextrose production |
| Storage | Store in cool, dry, sealed bags away from moisture. Protect from insects and rodents. Non-hazardous. |

| 102. Liquid Glucose (Corn Syrup) [Food Sweetener / Industrial Binder] |  |
| --- | --- |
| Chemical Formula | Starch hydrolysate (glucose + maltose + dextrins) |
| HS Code | 17023010 |
| CAS Number | 8029-43-4 |
| Appearance | Clear to slightly yellow viscous liquid |
| Odour | Faint sweet/caramel odour |
| Density | 1.37–1.42 g/cm³ (at 20°C) |
| Boiling Point | >100°C |
| Melting Point | N/A |
| Flash Point | >200°C |
| Solubility | Fully miscible with water |
| pH | 4.5–6.5 |
| Purity / Grade | DE 42 or DE 63 (dextrose equivalent) typically |
| Packaging | 280 kg drums / IBC / bulk tankers |
| Description | Liquid Glucose is a viscous sugar syrup obtained by enzymatic or acid hydrolysis of starch. It is used in confectionery, baking, and brewing as a sweetener and humectant, and in industrial applications as a binder and fermentation substrate. |
| Applications | Confectionery (candy, toffee, fondant); bakery (moisture retention); brewery & fermentation; pharmaceutical syrup base; adhesives & paper coating; tobacco humectant |
| Storage | Store in sealed containers at ambient temperature away from strong acids, alkalis, and fermentation conditions. |

| 103. Talc [Filler / Lubricant / Cosmetic Ingredient] |  |
| --- | --- |
| Chemical Formula | Mg₃Si₄O₁₀(OH)₂ |
| HS Code | 25262000 |
| CAS Number | 14807-96-6 |
| Appearance | White to grey-white fine, greasy-feeling powder |
| Odour | Odourless |
| Density | 2.58–2.83 g/cm³ (true) |
| Boiling Point | N/A |
| Melting Point | >1000°C |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water and organic solvents |
| pH | 8.5–10.0 (aqueous slurry) |
| Purity / Grade | ≥98% talc mineral |
| Packaging | 25 kg bags |
| Description | Talc is a soft, hydrophobic mineral (mohs hardness 1) used as a filler, lubricant, and extender in plastics, paper, paint, rubber, cosmetics, and pharmaceutical tablets. |
| Applications | Cosmetic (face powder, body powder, talcum powder); plastic & rubber filler (polypropylene, PVC); paper filler & coating; paint extender; pharmaceutical tablet lubricant; ceramic raw material |
| Storage | Store in sealed bags in dry conditions. Fine particle grades — use respiratory protection. Ensure asbestos-free certification for human-contact applications. |

| 104. Kaolin (China Clay) [Filler / Pigment Extender / Industrial Mineral] |  |
| --- | --- |
| Chemical Formula | Al₂Si₂O₅(OH)₄ |
| HS Code | 2507 |
| CAS Number | 1332-58-7 |
| Appearance | White to off-white fine powder |
| Odour | Odourless |
| Density | 2.58–2.63 g/cm³ |
| Boiling Point | N/A |
| Melting Point | >1700°C |
| Flash Point | Non-flammable |
| Solubility | Insoluble in water |
| pH | 5.5–8.0 |
| Purity / Grade | ≥98% kaolinite |
| Packaging | 25 kg bags / bulk |
| Description | Kaolin (China Clay) is a soft, white aluminosilicate mineral used as an extender/filler in paper, paints, plastics, ceramics, and rubber, and as an active ingredient in antidiarrheal pharmaceuticals. |
| Applications | Paper coating & filling; paint pigment extender; ceramic raw material; rubber/plastic filler; pharmaceutical antidiarrheal; cosmetic ingredient; pesticide carrier |
| Storage | Store in sealed bags in dry conditions. Non-hazardous under normal handling. |

| 105. Petroleum Jelly (Petrolatum) [Emollient / Cosmetic / Industrial Lubricant] |  |
| --- | --- |
| Chemical Formula | Complex mixture of refined mineral waxes and oils |
| HS Code | 27121090 |
| CAS Number | 8009-03-8 |
| Appearance | White to pale yellow translucent semi-solid |
| Odour | Odourless (white USP grade) |
| Density | 0.820–0.865 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | 40–60°C (depending on grade) |
| Flash Point | >200°C |
| Solubility | Insoluble in water; soluble in chloroform, benzene, and other non-polar solvents |
| pH | N/A |
| Purity / Grade | White USP/BP grade; Technical grade |
| Packaging | 25 kg tins / 180 kg drums |
| Description | Petroleum Jelly (Petrolatum, Vaseline) is a refined mixture of semi-solid hydrocarbons used as a skin protectant, emollient, lubricant, and corrosion inhibitor across cosmetics, pharmaceuticals, and industry. |
| Applications | Cosmetic & pharmaceutical emollient/moisturizer; baby care (diaper rash); wound care/healing ointment; industrial lubricant & rust preventive; electrical insulator; cosmetics base |
| Storage | Store in sealed containers at ambient temperature. Keep away from open flame — combustible at elevated temperatures. |

| 106. White Mineral Oil (Liquid Paraffin) [Cosmetic / Food / Pharmaceutical / Industrial Lubricant] |  |
| --- | --- |
| Chemical Formula | Complex saturated hydrocarbon mixture (CnH2n+2) |
| HS Code | 27101989 |
| CAS Number | 8042-47-5 |
| Appearance | Colourless, transparent oily liquid |
| Odour | Odourless (white grade) |
| Density | 0.840–0.905 g/cm³ |
| Boiling Point | >300°C |
| Melting Point | N/A |
| Flash Point | >150°C (light grade); >200°C (heavy grade) |
| Solubility | Insoluble in water; miscible with most non-polar solvents |
| pH | N/A |
| Purity / Grade | Meets USP/BP specifications (pharmaceutical grade); food grade (FDA/EC compliant) |
| Packaging | 200 L drums / IBC |
| Description | White Mineral Oil (Liquid Paraffin) is a highly refined, colourless, tasteless, and odourless mineral oil used as a laxative, emollient, food lubricant (food-contact machinery), and base oil in cosmetics and pharmaceuticals. |
| Applications | Pharmaceutical laxative (oral); cosmetic emollient & skin moisturizer; food-contact machinery lubricant (H1 grade); animal feed supplement; personal care formulations; release agent in food processing |
| Storage | Store in sealed containers away from heat sources. Chemically inert under normal storage conditions. |

| 107. Toluene Diisocyanate (TDI) [Polyurethane Raw Material / Isocyanate] |  |
| --- | --- |
| Chemical Formula | C₉H₆N₂O₂ (80/20 or 65/35 isomer mixture) |
| HS Code | 292910 |
| CAS Number | 584-84-9 |
| Appearance | Colourless to pale yellow fuming liquid |
| Odour | Pungent, sharp, irritating |
| Density | 1.213–1.225 g/cm³ |
| Boiling Point | 251°C |
| Melting Point | 11°C (80:20 TDI) |
| Flash Point | 132°C |
| Solubility | Reacts with water; miscible with organic solvents |
| pH | N/A |
| Purity / Grade | ≥99.5% |
| Packaging | 250 kg drums / ISO tanks |
| Description | Toluene Diisocyanate (TDI) is a key raw material for flexible polyurethane foam, coatings, adhesives, and sealants. It reacts with polyols to form polyurethane polymers with controlled properties. |
| Applications | Flexible polyurethane foam (furniture, mattresses, automotive seating); polyurethane coatings; adhesives & sealants; elastomers; binders (foundry sand) |
| Storage | Moisture-sensitive — store in sealed, dry containers. Highly toxic respiratory hazard — full respiratory PPE required. Reacts vigorously with water, alkalis, and alcohols. |

| 108. Soap Noodles [Soap Raw Material] |  |
| --- | --- |
| Chemical Formula | Sodium soap of fatty acids (C12–C18 saturated & unsaturated) |
| HS Code | 34012000 |
| CAS Number | Mixture |
| Appearance | White to cream-coloured cylindrical pellets/noodles |
| Odour | Faint characteristic soapy odour |
| Density | 0.8–0.9 g/cm³ (bulk) |
| Boiling Point | N/A |
| Melting Point | 40–50°C |
| Flash Point | >200°C |
| Solubility | Dispersible/emulsifiable in hot water |
| pH | 9.0–11.0 (1% solution) |
| Purity / Grade | Total fatty matter (TFM): 76–80% (76 TFM) or 80% (80 TFM) |
| Packaging | 25 kg bags / cartons |
| Description | Soap Noodles are semi-processed soap base pellets produced by saponification of palm/coconut oil blends with caustic soda. They are the primary raw material for bar soap manufacturing worldwide. |
| Applications | Bar soap manufacturing (bath soap, laundry soap); toilet soap; medicated soap; hotel amenity soap; glycerin-based transparent soap |
| Storage | Store in cool, dry area away from direct sunlight. Protect from moisture. Non-hazardous under normal handling. |

| 109. Methylene Chloride (DCM) [Chlorinated Solvent] |  |
| --- | --- |
| Chemical Formula | CH₂Cl₂ |
| HS Code | 29031200 |
| CAS Number | 75-09-2 |
| Appearance | Colourless, volatile liquid |
| Odour | Sweet, chloroform-like odour |
| Density | 1.325 g/cm³ |
| Boiling Point | 39.6°C |
| Melting Point | -97°C |
| Flash Point | Non-flammable (but forms explosive vapour-air mixtures above 25°C) |
| Solubility | 20 g/L (20°C) in water; miscible with most organic solvents |
| pH | N/A |
| Purity / Grade | ≥99.5% |
| Packaging | 250 kg drums |
| Description | Methylene Chloride (Dichloromethane, DCM) is a fast-evaporating chlorinated solvent widely used as a paint and varnish stripper, extraction solvent in pharmaceuticals and food processing, and solvent for adhesives and coatings. |
| Applications | Paint & coating stripper; pharmaceutical extraction (caffeine, APIs); foam blowing agent; adhesive & sealant solvent; aerosol propellant; metal degreasing |
| Storage | Store in sealed containers away from heat. Non-flammable but vapours can displace oxygen in confined spaces. Use respiratory protection. Suspected carcinogen — follow local regulations. |

| 110. Dextrose Monohydrate [Food / Pharmaceutical / Fermentation Ingredient] |  |
| --- | --- |
| Chemical Formula | C₆H₁₂O₆·H₂O |
| HS Code | 17023010 |
| CAS Number | 5996-10-1 |
| Appearance | White, crystalline, free-flowing powder |
| Odour | Odourless to faint sweet |
| Density | 1.54 g/cm³ |
| Boiling Point | N/A (decomposes on heating) |
| Melting Point | 83°C (loses water); anhydrous ~146°C |
| Flash Point | Non-flammable |
| Solubility | 91 g/100 mL (20°C) in water |
| pH | 3.5–5.5 (10% solution) |
| Purity / Grade | ≥99.5% (food grade); ≥99.5% (pharmaceutical grade USP) |
| Packaging | 25 kg bags / 50 kg bags |
| Description | Dextrose Monohydrate (glucose monohydrate) is the crystalline form of D-glucose widely used in food production, intravenous pharmaceutical solutions, fermentation media, and as a sweetener and energy source. |
| Applications | Food sweetener, energy drink ingredient; pharmaceutical IV glucose solution; fermentation carbon source; bakery (yeast food, browning); sports nutrition; dialysis solution |
| Storage | Store in sealed bags in cool, dry conditions away from moisture. Non-hazardous under normal conditions. |

| 111. Citric Acid [Organic Acid / Food Additive / Chelating Agent] |  |
| --- | --- |
| Chemical Formula | C₆H₈O₇ |
| HS Code | 29181400 |
| CAS Number | 5949-29-1,77-92-9 |
| Appearance | White crystalline powder (monohydrate or anhydrous) |
| Odour | Odourless |
| Density | 1.665 g/cm³ (anhydrous) |
| Boiling Point | Decomposes at 175°C |
| Melting Point | 153°C (anhydrous); 100°C (monohydrate, loses water) |
| Flash Point | Non-flammable |
| Solubility | 73 g/100 mL (20°C) in water; soluble in ethanol |
| pH | ~2.2 (1% solution) |
| Purity / Grade | ≥99.5% (food grade); ≥99.5% (pharmaceutical grade BP/USP) |
| Packaging | 25 kg bags |
| Description | Citric Acid is the most widely used acidulant and flavouring agent in food and beverages globally, and serves as a pH adjuster, antioxidant synergist, chelating agent, and preservative across food, pharmaceutical, personal care, and industrial applications. |
| Applications | Food & beverage acidulant (E330); pharmaceutical effervescent tablets & pH adjuster; cosmetic pH modifier & chelating agent; household cleaners & descalers; industrial water treatment; detergent builder |
| Storage | Store in sealed bags in cool, dry conditions. Hygroscopic — protect from moisture. Non-hazardous under normal handling. |

| 112. Cetostearyl Alcohol (CSA) [Cosmetic Co-emulsifier / Thickener / Emollient] |  |
| --- | --- |
| Chemical Formula | C₁₆H₃₃OH + C₁₈H₃₇OH (cetyl & stearyl alcohol mixture) |
| HS Code | 38237090 |
| CAS Number | 67762-27-0 |
| Appearance | White waxy solid flakes or granules |
| Odour | Faint fatty odour |
| Density | 0.810–0.830 g/cm³ (at 60°C, liquid) |
| Boiling Point | >300°C |
| Melting Point | 48–56°C |
| Flash Point | >200°C |
| Solubility | Insoluble in water; soluble in ethanol and oils |
| pH | N/A |
| Purity / Grade | ≥97% combined cetyl + stearyl alcohol |
| Packaging | 25 kg bags |
| Description | Cetostearyl Alcohol (Cetearyl Alcohol) is a mixture of cetyl and stearyl fatty alcohols widely used as a co-emulsifier, emulsion stabilizer, thickener, and emollient in cosmetics and pharmaceuticals. It improves the feel, texture, and consistency of creams and lotions. |
| Applications | Cosmetic O/W emulsion thickener & stabilizer; creams, lotions, ointments; hair conditioner (co-conditioning agent); pharmaceutical cream/ointment base; emulsifying wax component |
| Storage | Store in sealed bags or drums below 30°C. Melts above 55°C — store away from heat sources. |